## APPENDIX

## Indicator 3.4: Healthcare Sector Emissions

DATA

This analysis utilized EXIOBASE 3.9.6, a global multi-regional environmentally extended input-output (EE-MRIO) database covering 44 countries and 5 rest-of-world (RoW) regions, with EXIOBASE tables in euros (Stadler et al. 2018). The most recent version of EXIOBASE, v 3.9. is based on supply and use tables up to 2020, with now-casted data for 2021 and 2022. Now-casted data must be used with caution, and it is not recommended to use nowcasted data for sector-level analysis, especially as nowcast predictions fail to adequately capture the impacts of the global pandemic. For this reason, this analysis uses the 2020 model for subsequent year calculations, and 2021-2023 expenditures were deflated accordingly.

National health expenditure data come from the Global Health Expenditure Database (GHED), and this analysis uses the December 12, 2025 version. For 2021–2023, expenditures must be expressed in 2020 USD to be compatible with the 2020 EXIOBASE model used for those years. This is accomplished through a three-step procedure using GHED-supplied exchange rates (‘xrt’, in national currency units per USD) and GDP price deflators (‘gdpd’, indexed to 2023 = 100). Subscripts t and 2020 denote values for the nominal year and the base year, respectively.

- Convert nominal-year USD expenditures to local currency.

che_pc_localₜ = che_pc_usdₜ * xrtₜ

che_localₜ = che_usdₜ * xrtₜ

- Deflate local currency expenditure to 2020 real values using the GDP price index.

che_pc_local₂₀₂₀ = che_pc_localₜ * (gdpd₂₀₂₀ / gdpdₜ)

che_local₂₀₂₀ = che_localₜ * (gdpd₂₀₂₀ / gdpdₜ)

- Convert 2020 real local currency back to 2020 USD using 2020 exchange rate.

che_pc_usd₂₀₂₀ = che_pc_local₂₀₂₀ / xrt₂₀₂₀

che_usd₂₀₂₀ = che_local₂₀₂₀ / xrt₂₀₂₀

The resulting che_pc_usd₂₀₂₀ and che_usd₂₀₂₀ values represent real 2020 USD expenditures and are used in place of the nominal GHED values when calculating emissions and DALYs for 2021–2023.

METHODS

Emissions Calculations

This indicator is in the form of healthcare-associated GHG emissions per capita per year, including direct emissions from healthcare facilities as well as emissions from the consumption of goods and services supplied by other sectors. Results are calculated by assigning aggregate national health expenditures from WHO to final demand for ‘Health and Social Work’ sectors in the EE-MRIO model.  Environmental satellite accounts accompany each EE-MRIO model. Consumption-based GHG emissions are then calculated using the standard Leontief inverse technique (Miller and Blair 2009).

The Leontief inverse is calculated as L = (I-A)-1.

Where:

- I = identity matrix.

- A = technical coefficient matrix.

Unlike EXIOBASE version 3.8 which provided precalculated environmental account matrices, including multipliers, version 3.9 provides the F matrix (factor of production matrix) per satellite account. We used the ‘air emissions’ satellite account for this analysis. F represents the direct environmental pressures directly attributable to the intermediate production activities of industries. The direct stressor intensity coefficients matrix, S, is calculated by normalizing F by economic output, where x represents total industry output.

S = Fx-1

The multiplier matrix M, representing the total (direct plus indirect) environmental requirements per unit of final demand, and is then calculated by:

M = SL

Where:

- S = factor of production coefficients by sector.

- L = Leontief inverse matrix.

Characterization factors were applied to M to transform individual stressor flows into midpoint or endpoint impact indicators, specifically GHG emissions (GWP100) and Human Health Impacts (DALYs) from particulate matter and photochemical ozone formation. Following recommendations by the EXIOBASE team, we used IPCC GWP factors for GHG emissions (Greenhouse Gas Protocol 2024), and LC-IMPACT characterization factors for human health impacts from particulate matter formation and photochemical ozone formation (Verones et al. 2016). This characterization step produces a characterized multiplier matrix with columns corresponding to industry-region combinations and rows corresponding to impact categories.

Impact coefficients for GHG emissions (GWP100) were extracted from the calculated M matrix, now representing characterized emissions in units of kg CO₂eq per Million EUR of final demand for the 'Health and Social Work' sector. Similarly, impact coefficients for health impacts attributable to PM2.5 and ozone exposure, expressed as disability-adjusted life years (DALYs), were extracted from the EXIOBASE 3.8.2 M matrix in units of DALYs per Million EUR of final demand.

For the 44 countries explicitly modelled in EXIOBASE, country-specific emission multipliers were assigned directly. For countries not individually represented in EXIOBASE, rest-of-world regional multipliers were assigned according to WHO regional classifications: South-East Asia and Western Pacific Region, European Region, African Region, Region of the Americas, and Eastern Mediterranean Region.

To calculate healthcare sector emissions and DALYs, the extracted impact coefficients were multiplied by national health expenditures with appropriate currency conversion. Calculations can be expressed by the following formulas:

Per capita GHG Emissions (kg CO₂eq) = GHG_coeff * pc_che_usd * CF / 1,000,000

Total GHG emissions (Mt CO₂eq) = Per capita GHG Emissions * (pop * 1,000) / 1,000,000,000

Total PM2.5 DALYs = PM2.5_DALYs_coeff * che_usd * CF

Total Ozone DALYs = Ozone_DALYs_coeff * che_usd * CF

Total DALYs = Total PM2.5 DALYs + Total Ozone DALYs

Where:

- GHG_coeff = Impact coefficient in units of kg CO₂eq/Million EUR

- DALYs_coeff = Impact coefficient in units of DALYs/Million EUR

- pc_che_usd = Current Health Expenditure (CHE) per Capita in US$

- che_usd = Current Health Expenditure (CHE), in million current US$

- CF = USD > EUR economic adjustment factor for data year, pulled from native WHO data for Eurozone countries.

- Pop = Population (in thousands)

Independent research by Pichler et al. on CO2 emissions (excluding other GHGs) associated with health care in OECD countries considered temporal trends and introduced adjustments into the emissions satellite accounts of the EE-MRIO model EORA to reflect shifts in major GHG emissions sources that occurred between the baseline model year and when each healthcare expenditure occurred.  Based on this approach, for years past the latest model year (2021-2023), emissions were updated to reflect the nominal year using the PRIMAP dataset –containing historical greenhouse gas emissions data for countries and sectors (Gütschow et al. 2025). Expenditures calculated with the 2020 model were multiplied by the growth rates from baseline year (2020) to nominal year from the corresponding sector, extracted from PRIMAP data. For the few countries where PRIMAP data did not exist, the original, unadjusted values were used. Botswana’s PRIMAP data showed highly anonymous growth rates, so unadjusted values were used

Emissions Disaggregation

For the most recent year, 2023, national health sector emissions were disaggregated by Greenhouse Gas Protocol scopes. Scope 1 emissions include direct emissions by health care facilities, Scope 2 emissions include emissions from purchased electricity, and Scope 3 emissions encompass emissions from the entire upstream supply chain. Scope calculations were derived using the following logic. Scope 1 is simply this characterized S matrix itself representing only the direct emissions of the health sector per dollar of its own output, with no supply chain considered. Scope 2 is calculated by isolating only electricity-producing sectors in the model, then asking: how much of the health sector's spending flows to those electricity sectors, and what are the emissions associated with that electricity production? Scope 3 is then derived as the remainder: the full supply-chain multiplier minus Scope 1 and Scope 2.

National health sector emissions were also disaggregated by within region, and outside region. Within region emissions represent emissions that occur inside the same country as the health system, or domestic supply chains and direct health system operations. Outside region emissions represent emissions that occur in other countries, where the national health system relies on imported goods and services. Within/outside region emission are calculated as follows. For each country's health sector, one column of the Leontief inverse is extracted. This column encodes how much production activity is triggered, across every sector in every country, when that health system spends one dollar. A binary mask is then applied to that column to separate rows belonging to the same country (within) from rows belonging to all other countries (outside). Multiplying the raw stressor intensity matrix S by each masked portion of the Leontief column yields two vectors of emissions intensities, one for emissions physically occurring inside the country, one for emissions occurring in other countries' production systems.

B

Notes:

GHED release

A new GHED version is released each year, often with small updates to historical country expenditure data. This release version contained significant revisions that altered global health spending trends seen in the last version. China’s 2022 health expenditure data, che_usd, was revised by 12.3%, while 2021 remained unchanged. Due to the weight of China’s expenditures, this changes the decrease in global in health spending from 2021 -2022 witnessed in the last release version – to an increase. USA had revisions of 0.7% for 2021 and 1.2% for 2022, which also impacts the global aggregate due to the important weight of the US in global health expenditure

EXIOBASE model

Compared to results calculated with the previous version of EXIOBASE (v3.8), country footprints largely see reductions across all years. One of the key updates in v3.9 is a new energy accounting procedure. On this new method, Rasul et al. (2024) states that the new procedure sees drastically fewer outliers in the form extremely high energy use coefficients. For example, Rasul et al. states that “more than 40% of the original coefficients are larger than its corresponding new coefficients by a factor of 2”. The new energy allocation procedure in v3.9 produces more realistic coefficients, multipliers, and footprints compared to v3.8, with greater internal consistency.

Other Caveats

As only total health expenditure data are available from WHO, all expenditures are assigned to Final Demand, with no separation for investment.

MRIO models are built from aggregated top-down statistical data.  Results do not reflect individual health care systems’ power purchase agreements for renewable energy or any offsetting activities.  Results do not include direct emissions of waste anaesthetic gases from clinical operations nor emissions from metered dose inhalers, as these are not currently reported consistently in national emissions inventories.

# References

Gütschow, Johannes ; Busch, Daniel ; Pflüger, Mika (2025): The PRIMAP-hist national historical emissions time series (1750-2024) v2.7. Zenodo. https://doi.org/10.5281/zenodo.17090760

Greenhouse Gas Protocol. (2024, August 7). IPCC global warming potential values (Version 2.0). https://ghgprotocol.org/sites/default/files/2024-08/Global-Warming-Potential-Values%20(August%202024).pdf

Miller RE, Blair PD. Input-output analysis: foundations and extensions. Cambridge, UK: Cambridge University Press; 2009.

Pichler P-P, Jaccard IS, Weisz U, Weisz H. International comparison of health care carbon footprints. Environmental Research Letters 2019; 14(6): 064004.

Rasul, K., Schmidt, S., Hertwich, E. G., & Wood, R. (2024). EXIOBASE energy accounts: Improving precision in an open-sourced procedure applicable to any MRIO database. Journal of Industrial Ecology, 28(6), 1771–1785. https://doi.org/10.1111/jiec.13563

Stadler K, Wood R, Bulavskaya T, et al. EXIOBASE 3: Developing a time series of detailed environmentally extended multi‐regional input‐output tables. Journal of Industrial Ecology 2018; 22(3): 502-15.

Verones, F. et al. (2016). LC-IMPACT version 0.5: A spatially differentiated life cycle impact assessment approach. https://www.lc-impact.eu/doc/LC-Impact_report_SEPT2016_20160927.pdf

WHO. Global Health Expenditure Database: Indicators and data. Geneva, Switzerland: World Health Organization; 2025.